MATCH MARKING REMOVAL OF BRIDGES	9/2/04

1.0  Description.  This work shall include the additional work required to match mark, dismantle and store the superstructure of the bridge at location as noted on the plans or as directed by the engineer.

2.0  Removal Requirements.  The contractor shall not proceed with the removal of this bridge until directed by the engineer.  The contractor shall conduct removal operations in a manner that will avoid further deterioration of the existing structure.

2.1  The various structural members shall be match marked with paint before dismantling to provide for an exact and expedient reassembly.  Each structural member shall be clearly painted with the bridge number for ready identification.  A match marking diagram for the bridge superstructure shall be prepared by the contractor and submitted to the engineer for approval.

2.2  The contractor’s dismantling operations of the superstructure shall not damage any of the structural members.  Removal of the rivets or bolts for disconnection of the structural members shall be done in a manner to avoid structural damage of the members, plates, clip angles or other appurtenances.  Any breakage or bending of the structural members during dismantling, removal, hauling and storage shall be repaired or replaced by the contractor as approved by the engineer at the contractor's expense.

3.0  Method of Measurement.  No measurement will be made.

4.0  Basis of Payment.

4.1  Payment for the above described work, including salvage operations, match marking, equipment, labor and any other incidental work necessary to complete this item, will be considered completely covered by the contract lump sum price for “Match marking and Storing Superstructure”.  Payment for removing the remaining part of the bridge shall be in accordance with removal of bridges in Sec 216.

4.2  If salvage is not required as directed by the engineer, payment for removing all of the bridge shall be in accordance with removal of bridges in Sec 216.  No payment will be made for match marking and storing superstructure.