REMOVAL OF CATHODIC PROTECTION SYSTEM	9/2/04

1.0  Description.  This work shall consist of disengaging the power to the cathodic protection system during rehabilitation.  The removal of the cathodic protection system shall include all of the cathodic protection system except as shown on the plans.

2.0  Removal Requirements.  Any damage cause by the contractor's operations to the cathodic protection system that is to remain in place, shall be repaired or replaced at the contractor's expense.

2.1  The existing cathodic protection system shall be turned off as directed by the engineer prior to beginning work.  The existing cathodic protection system shall not be reused except as shown on the plans.  All insulated wiring associated with the removal of the cathodic protection system shall be disposed by the contractor and as approved by the engineer.

2.2  If during removal of the cathodic protection system, the existing concrete deck is damaged and not covered by new concrete, the areas on the deck shall be repaired with Class B-1 concrete in accordance with Sec 704.  If the concrete is damaged at other locations, the repair areas shall be made with a qualified special mortar.  The qualified special mortar shall be in accordance with Sec 704.

3.0  Method of Measurement.  No measurement will be made.

4.0  Basis of Payment.  Payment for the above described work, including all material, equipment, labor and any other incidental work necessary to complete this item, will be considered completely covered by the contract lump sum price for “Removal of Cathodic Protection System”.