Reviewers:  Use on stage construction only, mostly interstate jobs.

SURFACE SEALING LOW SLUMP CONCRETE		9/2/04

1.0  Description.  This provision allows surface sealing low slump concrete to be applied as last order of work.

2.0  Construction Requirements.  The surface of the low slump concrete shall be surface sealed in accordance with Sec 703.3.8 except that lanes may be opened to traffic after the low slump concrete has properly cured in accordance with Sec 505 and the sealant applied as a last order of work.  Any lanes open to traffic prior to surface sealing shall have foreign materials removed.  Surfaces that are sealed after each stage of construction shall have all vertical construction joints between stages protected from the surface sealant.  If asphalt roadway surface is adjacent to the new low slump concrete, the asphalt surface shall be protected from spillage of the sealant.

3.0  Method of Measurement.  No measurement will be made.

4.0  Basis of Payment.  Payment for the above describe work will be considered completely covered by the contract unit price for other items included in the contract.