Reviewers:  This special can be used for concrete filled steel grid deck repairs also by removing half in the title, method of measurement and basis of payment.

HALF CONCRETE FILLED STEEL GRID DECK REPAIRS		1/28/05

1.0  Description.  This work shall consist of removing deteriorated concrete in the steel grid deck when the depth of repair exceeds 1/2 inch (13 mm) below the steel grid surface, preparing the repair site, forming where required, placing and finishing new concrete.

2.0  Construction Requirements.

2.1  Removal Requirements.  Concrete removal shall be in accordance with the removal requirements in Sec 704.  Particular care shall be taken not to disturb or damage the steel grid members, pans or the grid’s encased sound concrete.  The exposed steel grid system shall be thoroughly cleaned by sand or hydro blasting to the satisfaction of the engineer.

2.2  Patch Preparation Requirements.  Patch preparation requirements for the repair areas shall be in accordance with Sec 704.

2.3  Placement of New Concrete.  Placement of the new concrete shall be in accordance with Sec 704.

3.0  Method of Measurement.  The extent of repair may vary from the estimated quantities, but the contract unit price shall prevail regardless of the variation.  Final measurement will not be made for preparation of the existing deck.  Half concrete filled steel grid deck repairs will be measured nearest square foot (0.1 m2).

4.0  Basis of Payment.  Accepted quantities will be paid for at the contract unit price for “Half Concrete Filled Steel Grid Deck Repairs”.