SUPERSTRUCTURE FURNISHED BY THE CONTRACTOR		3/1/04

Amend Sec 718 to include the following:

718.3.7  Superstructure Furnished by the Contractor.  The contractor shall furnish the complete superstructure.  The superstructure shall include the end bent cap beam units, the intermediate bent cap beam units, all individual superstructure units and attaching hardware.  The superstructure shall be shop fabricated into individual units for shipment and field erected by units with splice or connection plates attached to form a complete structure.

718.3.7.1  Cap Beam Units.  Each end bent cap beam unit, including channels, structural tube, cover plate, stiffeners, end plates and bottom bearing plates, shall be fabricated and fully assembled in the shop.  Each intermediate bent cap beam unit, including angles, cover plate, stiffeners, end plates and bottom bearing plates, shall be fabricated and fully assembled in the shop.  All holes and slots shall be drilled full size in the shop.

718.3.7.2  Superstructure Unit.  Each individual superstructure unit, including the open grid flooring, thrie beam rails, channels and posts, shall be fabricated and fully assembled in the shop.  The upper portion of the bearing shall be shop welded to the respective superstructure unit.  All holes and slots shall be shop drilled to full size.  The open steel grid flooring shall be the same depth throughout the bridge and the top edges of all members shall be serrated and on the same plane.  Trim bars shall be provided at the sides and ends of each individual superstructure unit.  The main beams, supplemental bars, cross bars and trim bars shall be factory welded at all joints.  Complete shop drawings for the open grid floor shall be prepared and submitted for approval in accordance with Sec 1080.

Delete Sec 718.5 and substitute the following:

718.5  Basis of Payment.  The substructure, furnishing superstructure, partial furnishing of superstructure, transporting and erecting superstructure, and removing and storing superstructure will be paid for at the contract unit price for each of the pay items included in the contract.  Payment will be considered full compensation for all labor, equipment and material, including attaching hardware, to complete the described work.  Any salvage value of the substructure shall be reflected in the contract unit price.